ITERATIVE DECODING AND ERROR CODE

CORRECTION METHOD IN HOLOGRAPHIC

DATA STORAGE

1 Optilink Ltd, Bánhida 24, 1116 Budapest, Hungary, attila.suto@axelero.hu;

Budapest University of Technology and Economics, Budafoki 8, 1111 Budapest, Hungary,

lorincz@eik.bme.hu

Abstract: We propose a technique for data detection in two-dimensional page-access optical

memory. The technique combines correlation decoding by the use of the

Reed- Solomon algorithm with decision feedback to improve the symbol-errorrate

performance in a system corrupted by intersymbol interference. By using

the addition information of Reed-Solomon decoder, it is possible to increase efficiency

of block decoding algorithm by correction the first order local errors in

the received block code. We find experimentally that the feedback decision can

improve data density by 11% during page recovery.

INTRODUCTION

We are developing a holographic memory system (HMS) based on polarization holography in thin-film photo-anisotropic polymer storage material . A spatial light modulator (SLM) as two-dimensional data page using sparsely constant weight modulation coding feeds data into the system . The information is stored in spatially filtered Fourier holograms. The coherent twodimensional image is reconstructed by a pixelated detector (CCD) that converts the coherent images into electric signals. The original information can be recovered after decoding and error correction coding (ECC). In data storage systems the data density is the most important parameter. To increase data density higher signal to noise ratio (SNR) is necessary. More data processing time and redundancy that is appended to the source data can increase the SNR. 1 and 2288 For optimum condition it is necessary to find a trade-off among data density, processing time and redundancy. A raw error ratio of is suitable for ECC to achieve the  cceptable user error rate. This value can be reached from different SNR in function of decoding method and processing time consumption. The SNR and processing time can be optimized in a given HMS. Conventional error-correction coding techniques  can be used to reduce the minimum signal-to-noise ratio that is required to achieve an acceptable symbol error rate and can therefore be used to increase the data density. Basically there are two approximations to solve the above-mentioned problem. First, the decoded data stream is feed into a Reed-Solomon ECC system  that can recover data through the noisy channel. This type of solution is effective when we have no information about the decoding defects. The second ECC method is the so-called Viterbi detection [5], which gives suitable result if the channel noise of the holographic system is known exactly. In fact, the real situation is that one part of noise comes from a well-described noise model; another part of noise may be estimated only during the decoding process. In the following we present data organization of our HMS and the idea of an iterative decoding and feedback ECC process. We outline experimental system and the results of performed measurements with different ECC and decoding methods. We show effectiveness of our iterative decoding with ECC  feedback to the achievable data density of the holographic system.

Figure 1. Schematic diagram of holographic memory system in the architecture.

2. CHANNEL MODEL

The schematic of a (focal length), telecentric HMS is shown in Fig. 1. Each of the two lenses in the system performs a Fourier-transform (FT) operation, leading to the SLM being imaged onto the CCD. The aperture in the FT plane helps to minimize the blockage of useful signal and to maximize the storage density. The storage medium is placed priori to the first FT plane so that we can record a hologram. The system uses a grid of input-plane SLM and Iterative Decoding and Error Code Correction Method. . . 89

pixels to represent binary 1’s and 0’s. We assume that the HMS is used in pixel-matched mode, wherein each SLM pixel is imaged onto one CCD pixel. Several factors affect the pixel values output of the CCD. The optical aperture and the contrast ratio are main important factors that affect the CCD output

value. Imaging imperfections of the optical system can be represented to the first order by the point-spread function (PSF). The PSF characterizes how much a point source spreads in consequence of the non-ideal imaging system. The width of PSF is inversely proportional to the width of aperture in the

frequency plane. If the input pattern in the object plane were a point source, the 2D field distribution in the output plane would correspond to the PSF. The PSF for the system with a square aperture of width would be Here is the wavelength. Interpixel cross talk, also called intersymbol interference (ISI), can arise

owing to the finite optical aperture and the limited modulation transfer function of the system. ISI is systematic in nature and is therefore predictable for given system parameters, whereas optical scattering and electric noises are random. There are two models to study the effects of an aperture in the  Fouriertransform plane of a imaging system. The first model is referred to as the convolution model. This model calculates the predicted spatial distribution at various planes in the imaging system by performing the appropriate convolutions of the input image distribution dictated by Fourier transform theory.

The second model is referred to as the fast Fourier-transform (FFT) model. This model also calculates the predicted spatial distribution of the signal at the hologram plane (aperture plane) but differs from the convolution model in that a two-dimensional FFT routine is used rather than convolution integral in the

computation. We used FFT model to study the above-mentioned holographic memory system, the model is useful for the inclusion of arbitrary data formats and apertures because it is not necessary to express these patterns in an analytic form, and because its execution requires relatively little computational time.

The two-dimensional FFT of the input plane (data page) was performed by using the Cooley-Tukey FFT algorithm. FFT’s were performed on each of the rows and then on the columns to give the complex-valued distribution of the Fourier transform plane. The effect of the aperture in the Fourier transform

plane was included by multiplying the Fourier-transform distribution by an aperture function, which consisted of pixels with amplitude one inside the aperture and amplitude zero outside the aperture diameter. Using this channel model it is possible to calculate the effect of spatial filtering and finite SLM

contrast with different 2D data pages. 90 and For a given aperture, SLM fill factor and noise level the Signal to Noise Ratio (SNR) is defined as where mean average value of the ON and OFF pixel distribution and mean their variance. It is desirable to know how the SNR as defined in Eq. 2 scales with symbol error rate (SER) for a holographic storage system where symbol means one code word created by the encoder. We present several methods in the following section to approve SER in function of SNR.

3. SOURCE ENCODING AND DECODING

Input data are converted to binary matrix using Look-up table (LUT). The purpose of a modulation code is to constrain the digital data patterns that are recorded. Some patterns are more likely to be corrupted by the channel, while other patterns may be particularly helpful for a given detection scheme. In our system we found optimal size of block code and code patterns. We implemented 4 by 4 pixel block code using constant ON pixel ratio. Each pattern contains three ON pixels in order to maximize signal to noise ratio in the holographic channel. After read-out data follow the decoding process that identifies block codes with correlation decoding. Let the noisy received code word be a vector of length x = y + n, where y is the transmitted code word and n is the noise vector imposed by the holographic recording-reconstruction process. An advantage of the constant weight modulation code (fixed ON pixels content per code word) is that we can implement Maximum Likelihood (ML) detection to find the most probable transmitted code word. Below we show how ML detection process helps us to choose correctly the minimum-distance valid code word. Using constant weight code, the decoding algorithm is simplified for the task of maximum searching. Eq. 3 defines the decoding process labelling with K(x, y) scalar function. where is the pattern in the Look-up table, x is vector representation of the detected pattern. The noise source generated errors can be eliminated using ECC. A Reed-Solomon code is arguably the most powerful tool in the family of block codes. The non-binary Reed-Solomon ECC is able to indicate the success of error correction. The ECC output is able to show defective patterns. The feedback of this information to correlation decoding is useful to find another pattern, which is the nearest to the previous pattern. The feedIterative Decoding and Error Code Correction Method.  back increases the efficiency of the ECC. Fig. 2 shows the process of iterative decoding using the feedback ECC information.

Figure 2. Data flow with feedback.

The ECC block error control code C consists of a set of M code words

Each code word is of the form

if the individual coordinates take on values from the LUT. The encoding process

consists of breaking up the data stream into blocks, and mapping these

blocks onto code word in C. Mathematical representation of decoding and ECC

can be written in the following way:

Where DEC function represents the decoding process formulated with Eq. 3.

ECC function name represents Reed Solomon block decoder process. In general

case this process can be implemented without any feedback decision.

However, the iterative feedback decision increases the effectiveness and noise

tolerance The feedback decision can be written mathematically in the following

way:

different code words in increasing order. The value depends on the ECC

feedback information, if RS algorithm indicates success during decoding then

else ECC decoding process is repeated while there is a successful ECC

indication or limited by fixed iteration step. Of course data transfer rate will

increase with the value. The advantage of the above-mentioned algorithm

is that the RS ECC with fixed parameter is more efficient than without the

Denote expression the result of the decoding

process with as correlation values of the decoding process for the

92 and

feedback decision method. It is important to find the trade-off between data

transfer rate and tolerance of the noisy channel during optimization of system

parameters.

Figure 3. SER in function of SNR and iteration steps.

4. SIMULATION RESULTS

We show decision algorithms for decoding method with feedback and without

feedback. By using the above-mentioned system model different decoding

methods could be compared. The system SNR is estimated by using FFT

model of the Fourier imaging system shown in Fig. 1. The output of the

simulation is the SER. The data page consists of 320×240 pixels with

pixel sizes. The data page is divided into 4 by 4 pixels, which represents one

code word. We used Fourier objective with a focal length of 4.4 mm. The

laser wavelength is 532 nm. During simulation we used interleaved RS encoder/

decoder with (60,52)(60,54) parameter.

Fig. 3 shows simulated SER in function of the SNR for different steps of

iteration. The is the result of correlation decoding and ECC without

feedback. The result shows that there is a range in the SNR where the feedback

decision is very efficient. In the SNR range of 2.5÷4 the value of SER

significantly improves in function of the SNR. It comes from the properties of

constrained modulation codes. In the beginning of third section we mentioned

that we use 4 by 4 pixels as code word. This block code contain only 8 bits

so there are several unused code words in the LUT. If the received code word

equals to an unused code word then it is easy to correct it by using feedback

decision. However, the received code word is in the LUT then more iteration

is required to get the right code word.

Iterative Decoding and Error Code Correction Method. . . 93

Figure 4. Data density in function of processing time consumption for target

5. EXPERIMENTAL RESULTS

The feedback decision was implemented on Texas Instrument’s

TMS320C6201 DSP processor with a Co-Optic Coic5130A RS

encoder/decoder and tested in our HMS.

We compared different methods of decoding and ECC performing experiments

with the HMS and determining data density and processing time for a

target SER of Fig. 4 shows how data density depends on the processing

time when using different decoding methods. The #0 means the result of the

global threshold method with ECC. The #1 is the result of the calculation using

local threshold decoding method with ECC. Local threshold method means

that the received code word is sorted in decreasing order and the bits associated

with the largest NON received values (constant weight code) are set to 1,

the remaining positions are set to 0. For #2 we used the correlation-decoding

method (see Eq. 3) without any feedback from the ECC. #3 shows the effect of

feedback ECC processing method. #4 presents the result with iterative

feedback ECC.

Comparing different methods can be concluded that the feedback decision

can increase the data density with about 11% in contrast to correlation decoding.

Whereas the iterative decoding with feedback seems capable of providing

as much as 18% density gain compared to simple feedback decision. In contrast

to simple threshold method, block based modulation codes improve data

density without significant processing time overhead.

94 and

6. CONCLUSION

We have shown a simple channel model for the holographic data storage system.

We used FFT algorithm for analysis and selection of system parameters

for Fourier-transform imaging system. Sparse modulation block code was

shown as an efficient way to construct 2-D data pages. We used small block

length so a look-up table can be an effective solution. We have shown that

the decoding algorithm provides maximum-likelihood performance in order to

constant weight coding.

We have described a feedback decision detector for use in ISI-corrupted

page-access Fourier holographic memories. The detection method is 2-D, employing

correlation decoding and Reed-Solomon decoder. The incorporation

of feedback improves the SER performance and data density.

We have shown through simulation and experiment that the novel decoding

method can operate at a reasonable SER in a system. We found that a factorof-

2 improvement in data density can be readily achieved in contrast to simple

threshold method with ECC.