Photonic Simulation Software Tools

Photonic simulation software (PSS)

In the domain of PSS, there are many tools from nano/micro-scale component simulation software such as OptiFDTD (finite-difference time-domain simulator), to large scale system software such as OptiSystem (optical communication system  simulation software). In between, there are many other specialized tools for designing waveguides, optical fiber, optical gratings, bulk optics components etc. Low level software such as OptiFDTD, simulate the propagation of optical fields through nano- to micro-scaled devices by directly solving Maxwell’s equations numerically [3]. Waveguide software such as OptiBPM, is a specialized software tool used to simulate waveguides and other similar optical devices which have light propagation predominantly in one direction  over large distances. OptiBPM is based on the beam propagation method (BPM). In essence, the BPM is a semi-analytical technique that solves an approximation of the wave equation [4]. OptiSystem is a simulation package that can design, test and optimize virtually any type of optical link in the physical layer of a wide range of optical networks   It is based on large collection of realistic models for components and sub-systems.

Methodology

There are 3 basic levels that PSS can introduced as an educational aid:

1. Lecture examples and visualization,

2. Student assignments and labs and

3. Graduate and post-graduate research.

The most basic use of PSS in education is to supplement classroom lectures. Most commercially available tools provide excellent visualization capabilities for input (system and structure layout) as well as output (graphs, tables etc.). This technique can be used even for students who may not have proficient computer skills since there is no requirement for the student to run the software. It can be used to introduce fundamental concepts of Maxwell’s equations to and high level designs of optical systems. Currently, the greatest increase in the use of PSS in education is in computer labs. In this scenario, professors create assignments and labs for students to solve using PSS. Traditionally, student assignments were often limited to problems that could be solved analytically in a few pages of handwritten calculations. While this is a very important component of learning optics, the limitation severely restricts the amount of problems that can be assigned. Even when the problems can be solved analytically, complex mathematics required for the solution can often obfuscate the underlying physics or engineering concepts that are the purpose of the assignment. Professional PSS have user interfaces that are very closely related to real world problems. In OptiFDTD the user, uses a graphical interface to draw the structure to be simulated. In OptiSystem, the user arranges and connects icons that represent real devices in a clearly visual way.

There are many benefits of using PSS in computer labs. Students have the options to perform virtual experiments instantly. Simulations provide an excellent way to build an intuitive understanding of the physical effects that occur. They provide instant feedback on “what if?” scenarios. This allows students to follow their own curiosity. Computer simulations in a lab environment provide a complimentary educational tool that lies between theory and physical experiments. On one hand, simulations can easily handle a large number of practical problems that are not possible to solve analytically. On the other hand, many more structures and system can be simulated than is practical to construct for real world experiments. In addition to this, computer simulations also provide instant visualization of many more physical qualities than can be obtained easily in experiments. As an example, users can view plots of the electric field inside a waveguide at any point in time; something that can not be done with a real device  For now, the most common use of PSS in education is at the graduate and post-graduate levels for research. For many years, simulation software has been recognized as an important tool for the design and development of new devices and systems. Device and system researchers can concentrate on concepts and design without spending effort to create the numerical software themselves. Experience with professional quality simulation tools is an asset in itself. Professional software tools are used by researchers in telecommunication and other high tech companies all around the world. Students that have experience with such software during their formal education have clear advantage over students that do not.

Vector nature of electromagnetic waves

Photonic simulation software can be used for the demonstration of fundamental optics phenomena such as simple reflection, refraction, absorption etc. The particular example considered here is based on the OptiFDTD software – a time-domain simulation tool numerically solving Maxwell’s equations. OptiFDTD is applied, in parallel with an analytical analysis,to demonstrate the vector nature of the electromagnetic (EM) waves during the reflection of a linearly polarized EM wave from a mirror (Fig. 1, left) positioned in free space. This is a 2D propagation problem in a plane defined by the Z (longitudinal)  and X (transverse) axes of a coordinate system (the Y direction is perpendicular to the X-Z plane and points towards the reader). For the sake of simplicity we consider the case where the input optical beam (positioned at X=3 μm with a Gaussian profile with a half-width of 2 μm) is linearly polarized and there is only one component of the electric field (Ey). Our goal is to provide a visual explanation of the relationship between the direction of EM power flow and the different non-zero components of the electric and magnetic fields. The power flow of an EM wave is described by the time averaged value of the Poynting vector S which is proportional to the vector product of the magnitude of the electric and magnetic fields: E × H. Sx is proportional to (EyHz – EzHy) and Sz is proportional to (ExHy – EyHx). Sy is equal to zero since the propagation is only in the X-Z plane. Since we consider a linearly polarized EM wave (Ey field component only), the two non-zero components of the Poynting vector take a simpler form: Sx ~ EyHz and Sz ~ EyHx..

Fig. 1 Schematic representation of the propagation geometry (left) and distribution of the Ey electric field magnitude (right).

Fig. 2 Visual representation of the distribution of i) the longitudinal (Sz) and transverse (Sx) components of the Poynting vector and ii) the magnitude Hx and the Hz-fields. The redirection of energy in the X-direction is associated with a transfer of energy from the Hx to the Hz component of the magnetic field.

Example 2 Attenuation-Limited Fiber Length

The next example is based on a lab exercise created by Dr. Warren Koontz [7]. This lab can be downloaded from Optiwave web, including the software, OptiPerformer [8], required to run it. The object of the lab is to calculate the attenuation-limited fiber length based on a power budget equation then to simulate and verify that it meets the performance objectives. The power budget equation states that the transmitted power minus the receiver sensitivity must be greater than or equal to

the sum of the power losses plus the power margin. P S AL L L M T R F C A −,

where PT is the transmitter power, SR is the receiver sensitivity, A is the fiber attenuation, LF is the fiber length, LC is the coupling loss, LA is the additional know losses and M is the power margin.

Fig.

Fig. 3 Screen capture of OptiPerformer. The layout is of a single one channel optical link.

Fig.

Fig. 4 Screen capture of the BER analyzer. A graph of the Q factor as a function of decision instant is shown with the eye diagram superimposed on it.

The OptiPerformer layout of the single channel link is shown in Fig. The layout is fixed but the user can modify the laser output, the fiber length, the attenuation coefficient of the fiber, the thermal noise of the PIN detector and the wavelength of the laser. OptiPerformer files are created using, OptiSystem. Much more complex system layouts than this one can be created. The output of the BER analyzer is shown in Fig. 4. This gives the user visual feedback on the quality of the signal as well as numerical analysis of the quality of the signal. This diagram is calculated for any input parameters the user specifies. In this case, the fiber length was set to 120 km. The resulting maximum Q factor is 7.42 which corresponds to a bit error rate of 1.03x10-12.