# Network architecture:

Network architecture is the design of a communications network. It is a framework for the specification of a network's physical components and their functional organization and configuration, its operational principles and procedures, as well as data formats used in its operation.

In telecommunication, the specification of a network architecture may also include a detailed description of products and services delivered via a communications network, as well as detailed rate and billing structures under which services are compensated.

The network architecture of the Internet is predominantly expressed by its use of the Internet Protocol Suite, rather than a specific model for interconnecting networks or nodes in the network, or the usage of specific types of hardware links.

## Contents: - 1 OSI Network Model - 1.1 Physical Layer - 1.2 Data Linking Layer - 1.3 Network Layer - 1.4 Transport Layer - 1.5 The Session Layer - 1.6 Presentation Layer - 1.7 Application Layer - 2 Distributed computing - 3 References

## OSI Network Model:

The Open Systems Interconnection model (OSI model) is a product of the Open Systems Interconnection effort at the International Organization for Standardization. It is a way of sub-dividing a communications system into smaller parts called layers. A layer is a collection of similar functions that provide services to the layer above it and receives services from the layer below it. On each layer, an instance provides services to the instances at the layer above and requests service from the layer below.

### Physical Layer

The Physical Layer defines the electrical and physical specifications for devices. In particular, it defines the relationship between a device and a transmission medium, such as a copper or optical cable. This includes the layout of pins, voltages, cable specifications, hubs, repeaters, network adapters, host bus adapters (HBA used in storage area networks) and more. Its main task is the transmission of a stream of bits over a communication channel.

### Data Linking Layer

The Data Link Layer provides the functional and procedural means to transfer data between network entities and to detect and possibly correct errors that may occur in the Physical Layer. Originally, this layer was intended for point-to-point and point-to-multipoint media, characteristic of wide area media in the telephone system. Local area network architecture, which included broadcast-capable multiaccess media, was developed independently of the ISO work in IEEE Project 802. IEEE work assumed sublayering and management functions not required for WAN use. In modern practice, only error detection, not flow control using sliding window, is present in data link protocols such as Point-to-Point Protocol (PPP), and, on local area networks, the IEEE 802.2 LLC layer is not used for most protocols on the Ethernet, and on other local area networks, its flow control and acknowledgment mechanisms are rarely used. Sliding-window flow control and acknowledgment is used at the Transport Layer by protocols such as TCP, but is still used in niches where X.25 offers performance advantages. Simply, its main job is to create and recognize the frame boundary. This can be done by attaching special bit patterns to the beginning and the end of the frame. The input data is broken up into frames.

### [edit] Network Layer

Main article: Network Layer

The Network Layer provides the functional and procedural means of transferring variable length data sequences from a source host on one network to a destination host on a different network, while maintaining the quality of service requested by the Transport Layer (in contrast to the data link layer which connects hosts within the same network). The Network Layer performs network routing functions, and might also perform fragmentation and reassembly, and report delivery errors. Routers operate at this layer—sending data throughout the extended network and making the Internet possible. This is a logical addressing scheme – values are chosen by the network engineer. The addressing scheme is not hierarchical. It controls the operation of the subnet and determine the routing strategies between IMP and insures that all the packs are correctly received at the destination in the proper order.

### [edit] Transport Layer

Main article: Transport Layer

The Transport Layer provides transparent transfer of data between end users, providing reliable data transfer services to the upper layers. The Transport Layer controls the reliability of a given link through flow control, segmentation/desegmentation, and error control. Some protocols are state and connection oriented. This means that the Transport Layer can keep track of the segments and retransmit those that fail. The Transport layer also provides the acknowledgement of the successful data transmission and sends the next data if no errors occurred. Some Transport Layer protocols, for example TCP, but not UDP, support virtual circuits provide connection oriented communication over an underlying packet oriented datagram network .Where it assures the delivery of packets in the order in which they were sent and assure that they are free of errors .The datagram transportation deliver the packets randomly and broadcast it to multiple nodes. Notes: The transport layer multiplexes several streams on to 1 physical channel.The transport headers tells which message belongs to which connnection.

### [edit] The Session Layer

Main article: Session Layer

This Layer provide a user interface to the network where the user negotiate to establish a connection ,the user must provide the remote address in with he want to contact. The operation of setting up a session between 2 process is called "Binding" in some protocols it is merged with the transport layer.

### [edit] Presentation Layer

Main article: Presentation Layer

The Presentation Layer establishes context between Application Layer entities, in which the higher-layer entities may use different syntax and semantics if the presentation service provides a mapping between them. If a mapping is available, presentation service data units are encapsulated into session protocol data units, and passed down the stack.This layer provides independence from data representation (e.g., encryption) by translating between application and network formats. The presentation layer transforms data into the form that the application accepts. This layer formats and encrypts data to be sent across a network. It is sometimes called the syntax layer.[citation needed]The original presentation structure used the basic encoding rules of Abstract Syntax Notation One (ASN.1), with capabilities such as converting an EBCDIC-coded text file to an ASCII-coded file, or serialization of objects and other data structures from and to XML.

### [edit] Application Layer

Main article: Application Layer

The Application Layer is the OSI layer closest to the end user, which means that both the OSI application layer and the user interact directly with the software application. This layer interacts with software applications that implement a communicating component. Such application programs fall outside the scope of the OSI model. Application layer functions typically include identifying communication partners, determining resource availability, and synchronizing communication. When identifying communication partners, the application layer determines the identity and availability of communication partners for an application with data to transmit.

## [edit] Distributed computing

Main article: distributed computing

In distinct usage in distributed computing, the term network architecture often describes the structure and classification of a distributed application architecture, as the participating nodes in a distributed application are often referred to as a network. For example, the applications architecture of the public switched telephone network (PSTN) has been termed the Advanced Intelligent Network. There are any number of specific classifications but all lie on a continuum between the dumb network (e.g., Internet) and the intelligent computer network (e.g., the telephone network). Other networks contain various elements of these two classical types to make them suitable for various types of applications. Recently the context aware network, which is a synthesis of two, has gained much interest with its ability to combine the best elements of both.

A popular example of such usage of the term in distributed applications, as well as PVCs (permanent virtual circuits), is the organization of nodes in peer-to-peer (P2P) services and networks. P2P networks usually implement overlay networks running over an underlying physical or logical network. These overlay network may implement certain organizational structures of the nodes according to several distinct models, the network architecture of the system.

Network architecture is a broad plan that specifies everything necessary for two application programs on different networks on an Internet to be able to work together effectively.

# Hybrid fibre-coaxial

From Wikipedia, the free encyclopedia

Jump to: navigation, search

|  | This article does not cite any references or sources. Please help improve this article by adding citations to reliable sources. Unsourced material may be challenged and removed. (August 2007) |
| --- | --- |

Hybrid fibre-coaxial (HFC) is a telecommunications industry term for a broadband network which combines optical fibre and coaxial cable. It was commonly employed globally by cable television operators since the early 1990s.

HFC network diagram

## Contents [hide] - 1 Description - 2 Transport over HFC network - 3 Competitive network technologies - 4 See also

## [edit] Description

The fibre optic network extends from the cable operators' master headend, sometimes to regional headends, and out to a neighbourhood's hubsite, and finally to a fibre optic node which serves anywhere from 25 to 2000 homes. A master headend will usually have satellite dishes for reception of distant video signals as well as IP aggregation routers. Some master headends also house telephony equipment for providing telecommunications services to the community. A regional or area headend/hub will receive the video signal from the master headend and add to it the Public, educational, and government access (PEG) cable TV channels as required by local franchising authorities or insert targeted advertising that would appeal to a local area. The various services are encoded, modulated and upconverted onto RF carriers, combined onto a single electrical signal and inserted into a broadband optical transmitter. This optical transmitter converts the electrical signal to a downstream optically modulated signal that is sent to the nodes. Fibre optic cables connect the headend or hub to optical nodes in a point-to-point or star topology, or in some cases, in a protected ring topology.

A fibre optic node has a broadband optical receiver which converts the downstream optically modulated signal coming from the headend/hub to an electrical signal going to the homes. Today, the downstream signal is a radio frequency modulated signal that typically begins at 50 MHz and ranges from 550 MHz to 1000 MHz on the upper end. The fibre optic node also contains a reverse/return path transmitter that sends communication from the home back to the headend. In North America, this reverse signal is a modulated radio frequency ranging from 5 to 42 MHz while in other parts of the world, the range is 5 to 65 MHz.

The optical portion of the network provides a large amount of flexibility. If there are not many fibre optic cables to the node, wavelength division multiplexing can be utilised to combine multiple optical signals onto the same fibre. Optical filters are used to combine and split optical wavelengths onto the single fibre. For example, the downstream signal could be on a wavelength at 1310nm and the return signal could be on a wavelength at 1550nm. There are also techniques to put multiple downstream and upstream signals on a single fibre by putting them at different wavelengths.

The coaxial portion of the network connects 25 to 2000 homes (500 is typical) in a tree-and-branch configuration off of the node. Radio frequency amplifiers are used at intervals to overcome cable attenuation and passive losses of the electrical signals caused by splitting or "tapping" the coaxial cable. Trunk coaxial cables are connected to the optical node and form a coaxial backbone to which smaller distribution cables connect. Trunk cables also carry AC power which is added to the cable line at usually either 60V or 90V by a power supply and a power inserter. The power is added to the cable line so that trunk and distribution amplifiers do not need an individual, external power source. From the trunk cables, smaller distribution cables are connected to a port of the trunk amplifier to carry the RF signal and the AC power down individual streets. If needed, line extenders, which are smaller distribution amplifiers, boost the signals to keep the power of the television signal at a level that the TV can accept. The distribution line is then "tapped" into and used to connect the individual drops to customer homes. These taps pass the RF signal and block the AC power unless there are telephony devices that need the back-up power reliability provided by the coax power system. The tap terminates into a small coaxial drop using a standard screw type connector known as an “F” connector. The drop is then connected to the house where a ground block protects the system from stray voltages. Depending on the design of the network, the signal can then be passed through a splitter to multiple TVs. If too many splitters are used to connect multiple TVs, the signal levels will decrease, and picture quality on analog channels of TVs past those splitters will go down requiring the use of a "drop" or "house" amplifier.

## [edit] Transport over HFC network

By using frequency division multiplexing, an HFC network may carry a variety of services, including analogue TV, digital TV (SDTV or HDTV), Video on demand, telephony, and high-speed data. Services on these systems are carried on Radio Frequency (RF) signals in the 5 MHz to 1000 MHz frequency band.

The HFC network can be operated bi-directionally, meaning that signals are carried in both directions on the same network from the headend/hub office to the home, and from the home to the headend/hub office. The forward-path or downstream) signals carry information from the headend/hub office to the home, such as video content, voice and internet data. The return-path or upstream signals carry information from the home to the headend/hub office, such as control signals to order a movie or internet data to send an email. The forward-path and the return-path are actually carried over the same coaxial cable in both directions between the optical node and the home. In order to prevent interference of signals, the frequency band is divided into two sections. In countries that have traditionally used NTSC System M, the sections are 52 MHz to 1000 MHz for forward-path signals, and 5 MHz to 42 MHz for return-path signals. Other countries use different band sizes, but are similar in that there is much more bandwidth for downstream communication instead of upstream communication.

Traditionally, since video content was sent only to the home, the HFC network was structured to be non-symmetrical: one direction has much more data-carrying capacity than the other direction. The return-path was originally only used for some control signals to order movies, etc., which required very little bandwidth. As additional services have been added to the HFC network, such as internet access and telephony, the return-path is being utilised more.

Multiple System Operators (MSOs) developed methods of sending the various services over RF signals on the fibre optic and coaxial copper cables. The original method to transport video over the HFC network and, still the most widely used method, is by modulation of standard analogue TV channels which is similar to the method used for transmission of over-the-air broadcast. See broadcast television system for more information. One analogue TV channel occupies a 6 MHz-wide frequency band in NTSC-based systems, or an 8 MHz-wide frequency band in PAL or SECAM-based systems. Each channel is centred on a specific frequency carrier so that there is no interference with adjacent or harmonic channels. Digital TV channels offer a more-efficient way to transport video by using MPEG-2 or MPEG-4 coding over Quadrature amplitude modulation (QAM) channels. To be able to view a digitally modulated channel, home, or customer-premises equipment (CPE), e.g. digital televisions, computers, or set-top boxes, are required to convert the RF signals to signals that are compatible with display devices such as analogue televisions or computer monitors. The Federal Communication Commission (FCC) has ruled that consumers can obtain a cable card from their local MSO to authorise viewing digital channels. By using digital compression techniques, multiple standard and high-definition TV channels can be carried on one 6 or 8 MHz frequency carrier thus increasing the channel carrying-capacity of the HFC network by 10 times or more versus an all analogue network. Note that a digital tuner (i.e. TV set-top box) is not required for standard analogue TV channels since most televisions have integrated analogue tuners that can decode the signal, unless some type of scrambling is used.

## [edit] Competitive network technologies

Digital subscriber line (DSL) is a technology used by traditional telephone companies to deliver advanced services (high-speed data and sometimes video) over twisted pair copper telephone wires. It typically has lower data carrying capacity than HFC networks and data speeds can be range-limited by line lengths and quality.

Satellite television competes very well with HFC networks in delivering broadcast video services. Interactive satellite systems are less competitive in urban environments because of their large round-trip delay times, but are attractive in rural areas and other environments with insufficient or no deployed terrestrial infrastructure.

Analogous to HFC, Fibre In The Loop technology is used by telephone local exchange carriers to provide advanced services to telephone customers over the POTS local loop.

In the 2000s, telecom companies started significant deployments of Fibre to the x such as passive optical network solutions to deliver video, data and voice to compete with cable operators. These can be costly to deploy but they can provide large bandwidth capacity especially for data services.

## [edit]

# Fiber to the x

From Wikipedia, the free encyclopedia

Jump to: navigation, search

FTTB, FTTC, FTTH, FTTK, FTTN, and FTTP all redirect here. For airports with those ICAO codes, see List of airports in Chad.

Fiber to the x (FTTx) is a generic term for any broadband network architecture that uses optical fiber to replace all or part of the usual metal local loop used for last mile telecommunications. The generic term originated as a generalization of several configurations of fiber deployment (FTTN, FTTC, FTTB, FTTH...), all starting by FTT but differentiated by the last letter, which is substituted by an x in the generalization.

A schematic illustrating how FTTx architectures vary — with regard to the distance between the optical fiber and the end-user. The building on the left is the central office; that on the right is one of the buildings served by the central office. Dotted rectangles represent separate living or office spaces within the same building.

## Contents [hide] - 1 Definition of terms - 2 Benefits of fiber in the access network - 3 Ethernet Point-to-Point - 4 Fiber to the node - 5 Fiber to the curb - 6 Fiber to the premises - 6.1 FTTH vs. FTTB - 7 Deployments - 7.1 FTTH - 7.2 FTTN - 7.3 Direct fiber - 7.4 Shared fiber - 7.4.1 Active optical network - 7.4.2 Passive optical network - 7.5 Electrical portion - 8 See also - 9 Notes and references - 10 External links

## [edit] Definition of terms

The telecommunications industry differentiates between several distinct configurations. The terms in most widespread use today are:

- FTTN - Fiber-to-the-node - fiber is terminated in a street cabinet up to several kilometers away from the customer premises, with the final connection being copper. Fiber-to-the-node is often seen as an interim step towards full FTTH and is currently used by telecoms service providers like AT&T, Deutsche Telekom, Telekom Austria and Swisscom to deliver advanced triple-play services.

- FTTC - Fiber-to-the-cabinet - this is very similar to FTTN, but the street cabinet is closer to the user's premises; typically within 300m.

- FTTB - Fiber-to-the-building or Fiber-to-the-basement - fiber reaches the boundary of the building, such as the basement in a multi-dwelling unit, with the final connection to the individual living space being made via alternative means.

- FTTH - Fiber-to-the-home - fiber reaches the boundary of the living space, such as a box on the outside wall of a home. Active Ethernet Point-to-Point is fast emerging as the optimum architecture for delivering advanced triple-play services over FTTH networks because there is no limit on the distance between an operator‘s central office (CO) and a subscriber’s home.[1][2]

- FTTP - Fiber-to-the premises - this term is used in several contexts: as a blanket term for both FTTH and FTTB, or where the fiber network includes both homes and small businesses.

To promote consistency, especially when comparing FTTH penetration rates between countries, the three FTTH Councils of Europe, North America and Asia-Pacific have agreed upon definitions for FTTH and FTTB.[3] The FTTH Councils do not have formal definitions for FTTC and FTTN.

It is worth pointing out that fiber to the telecom enclosure (FTTE) is not considered to be part of the FTTx group of technologies, despite the similarity in name. FTTE is a form of structured cabling typically used in the enterprise local area network, where fiber is used to link the main computer equipment room to an enclosure close to the desk or workstation.[4] Similarly, in fiber-to-the-desk a fiber connection is installed from the main computer room to a terminal at the desk.

## [edit] Benefits of fiber in the access network

The speeds of fiber optic and copper cables are both limited by length, but copper is much more sharply limited in this respect. For example, gigabit Ethernet runs over relatively economical category 5e, category 6, or augmented category 6 unshielded twisted pair copper cabling but only to 100 meters. However, over the right kind of fiber, gigabit ethernet can easily reach distances of tens of kilometers.

Even in the commercial world, most computers have copper communication cables. But these cables are short, typically tens of meters. Most metropolitan network links (e.g., those based on telephone or cable television services) are several kilometers long, in the range where fiber significantly outperforms copper. Replacing at least part of these links with fiber shortens the remaining copper segments and allows them to run much faster.

Fiber configurations that bring fiber right into the building can offer the highest speeds since the remaining segments can use standard Ethernet or coaxial cable. Fiber configurations that transition to copper in a street cabinet are generally too far from the users for standard Ethernet configurations over existing copper cabling. They generally use VDSL at (downstream) speeds of several tens of megabits per second.

Fiber is often said to be 'future proof' because the speed of the broadband connection is usually limited by the terminal equipment rather than the fiber itself, permitting at least some speed improvements by equipment upgrades before the fiber itself must be upgraded. Still, the type and length of employed fibers chosen, e.g. multimode vs single mode, are critical for applicability for future high gigabit connections.

## [edit] Ethernet Point-to-Point

Ethernet Point-to-Point is widely accepted as the optimum architecture for delivering bandwidth-heavy next generation triple (and quad) play (voice, video, data and mobile) services over both fiber and hybrid fiber coax [HFC] networks. Active Ethernet Point-to-Point uses dedicated fiber from an operator’s central office all the way to the subscribers’ home, while hybrid networks [often FTTN] use it to transport data via fiber to a node, and then to ensure the highest possible throughput speeds over last mile copper connections.

This approach has become increasingly popular in recent years with telecoms service providers in both North America AT&T, Telus, for example] and Europe's Fastweb, Telecom Italia, Telekom Austria and Deutsche Telecom, for example]. Search specialist Google has also looked into this approach, amongst others, as a way to deliver multiple services over open-access networks in the United States.[5]

## [edit] Fiber to the node

Fiber to the node (FTTN), also called fiber to the neighborhood or fiber to the cabinet (FTTCab),[6] is a telecommunication architecture based on fiber-optic cables run to a cabinet serving a neighborhood. Customers typically connect to this cabinet using traditional coaxial cable or twisted pair wiring. The area served by the cabinet is usually less than 1,500 m in radius and can contain several hundred customers. (If the cabinet serves an area of less than 300 m in radius then the architecture is typically called fiber to the curb.)[7]

Fiber to the node allows delivery of broadband services such as high speed Internet. High speed communications protocols such as broadband cable access (typically DOCSIS) or some form of DSL are used between the cabinet and the customers. The data rates vary according to the exact protocol used and according to how close the customer is to the cabinet.

Unlike the competing fiber to the premises technology, fiber to the node often uses the existing coaxial or twisted pair infrastructure to provide last mile service. For this reason, fiber to the node is less costly to deploy. In the long-term, however, its bandwidth potential is limited relative to implementations which bring the fiber still closer to the subscriber.

A variant of this technique for cable television providers is used in a hybrid fiber-coaxial (HFC) system. It is sometimes given the acronym FTTN for Fiber To The Last Amplifier when it replaces analog amplifiers up to the last one before the customer (or neighborhood of customers).

## [edit] Fiber to the curb

Fiber to the curb (FTTC) is a telecommunications system based on fiber-optic cables run to a platform that serves several customers. Each of these customers has a connection to this platform via coaxial cable or twisted pair.

Fiber to the curb allows delivery of broadband services such as high speed internet. High speed communications protocols such as broadband cable access (typically DOCSIS) or some form of DSL are used between the cabinet and the customers. The data rates vary according to the exact protocol used and according to how close the customer is to the cabinet.

FTTC is subtly distinct from FTTN or FTTP (all are versions of Fiber in the Loop). The main difference is the placement of the cabinet. FTTC will be placed near the "curb" which differs from FTTN which is placed far from the customer and FTTP which is placed right at the serving location.

Unlike the competing fiber to the premises (FTTP) technology, fiber to the curb can use the existing coaxial or twisted pair infrastructure to provide last mile service. For this reason, fiber to the curb costs less to deploy. However, it also has lower bandwidth potential than fiber to the premises.

In the United States of America and Canada, the largest deployment of FTTC was carried out by BellSouth Telecommunications. With the acquisition of BellSouth by AT&T, deployment of FTTC will end. Future deployments will be based on either FTTN or FTTP. Existing FTTC plant may be removed and replaced with FTTP.[8]

## [edit] Fiber to the premises

Fiber to the premises is a form of fiber-optic communication delivery in which an optical fiber is run from the central office all the way to the premises occupied by the subscriber. Fiber to the premises is often abbreviated with the acronym FTTP. However, this acronym has become ambiguous and may instead refer to a form of fiber to the curb where the fiber terminates at a utility pole without reaching the premises.

### [edit] FTTH vs. FTTB

Fiber to the premises can be categorized according to where the optical fiber ends:

- FTTH (fiber to the home) is a form of fiber optic communication delivery in which the fiber extends from the central office to the subscriber's living or working space.[9] Once at the subscriber's living or working space, the signal may be conveyed throughout the space using any means, including twisted pair, coaxial cable, wireless, power line communication, or optical fiber.

- FTTB (fiber to the building, also called fiber to the basement) is a form of fiber optic communication delivery in which the optical fiber terminates before actually reaching the subscribers living or working space itself, but does extend to the property containing that living or working space. The signal is conveyed the final distance using any non-optical means, including twisted pair, coaxial cable, wireless, or power line communication.[10] By definition, FTTB necessarily applies only to those properties which contain multiple living or working spaces.

An apartment building may provide an example of the distinction between FTTH and FTTB. If a fiber is run to a panel at each subscriber's apartment, this is FTTH. If instead the fiber goes only as far as the apartment building's shared electrical room, then this is FTTB.

## [edit] Deployments

#### [edit] FTTH

- Fastweb - Italian operator Fastweb launched the first commercial fiber-to-the-home service in 2001. Using an Active Ethernet Point-to-Point architecture, the service delivered voice, video and data services to thousands of subscribers’ homes in Italy over a 10MB symmetrical dedicated fiber connection. Fastweb used one of the first residential gateways for both multiple dwelling units [MDUs] as well as residential homes that provided embedded fiber-termination, designed and built by Advanced Digital Broadcast, to enable consumers to share services with a range of CE devices around the home.[11]

The deployment of an FTTH network meant Fastweb was the first telecom operator to deliver true triple-play services to its subscribers. This contributed to its ARPU [Average Revenue Per User] being amongst the highest in the industry for a number of years during the early 2000s. Its FTTH network also puts it at the forefront of advanced connected home services.

- Fiber for Italy - The Fiber For Italy[12] Initiative has the stated goal of offering 100MBps symmetrical connections to 10 million Italian subscribers across 15 cities by [2018] and up to 1GBps for business customers.[13] It involves operators Wind, Tele2, Vodafone and Fastweb. An ongoing pilot project in the Italian capital Rome delivers symmetrical speeds of up to 100MBps to small businesses. Italian state operator Telecom Italia is not a participant in the Fiber For Italy programme, but has independently committed to provide ultra-high speed broadband up to 100MBps symmetrical connections to 50 percent of the country’s population (138 cities) by 2018.[14]

Both Fiber for Italy participants and Telecom Italia are working with Advanced Digital Broadcast to provide residential gateway technology with embedded fiber termination.

#### [edit] FTTN

FTTN, or Fiber-to-the-node, is currently used by a number of multiple-service operators to deliver advanced triple play services to consumers, including AT&T in the United States for its U-Verse service, Deutsche Telekom in Germany, Swisscom and Canadian operator Telus. It is seen as an interim step towards full FTTH and in many cases services triple play services delivered using this approach has been proven to grow subscriber numbers and ARPU considerably.[15][16]

### [edit] Direct fiber

The simplest optical distribution network can be called direct fiber. In this architecture, each fiber leaving the central office goes to exactly one customer. Such networks can provide excellent bandwidth since each customer gets their own dedicated fiber extending all the way to the central office. However, this approach is about 10% more costly due to the amount of fiber and central office machinery required.[17] The approach is generally favored by new entrants and competitive operators. A benefit of this approach is that it doesn't exclude any layer 2 networking technologies, be they Passive optical network, Active Optical Network, etc. From a regulatory point of view it leads to least implications as any form of regulatory remedy is still possible using this topology.[18]

### [edit] Shared fiber

More commonly each fiber leaving the central office is actually shared by many customers. It is not until such a fiber gets relatively close to the customers that it is split into individual customer-specific fibers. There are two competing optical distribution network architectures which achieve this split: active optical networks (AONs) and passive optical networks (PONs).

#### [edit] Active optical network

Comparison showing how a typical active optical network handles downstream traffic differently than a typical passive optical network. The type of active optical network shown is a star network capable of multicasting. The type of passive optical network shown is a star network having multiple splitters housed in the same cabinet.

Active optical networks rely on some sort of electrically powered equipment in Optical Distribution Network(ODN) to distribute the signal, such as a switch or router. Normally, optical signals need O-E-O transformation in ODN. Each signal leaving the central office is directed only to the customer for which it is intended. Incoming signals from the customers avoid colliding at the intersection because the powered equipment there provides buffering.

As of 2007, the most common type of active optical networks are called active Ethernet, a type of Ethernet in the first mile (EFM). Active Ethernet uses optical Ethernet switches to distribute the signal, thus incorporating the customers' premises and the central office into one giant switched Ethernet network. Such networks are identical to the Ethernet computer networks used in businesses and academic institutions, except that their purpose is to connect homes and buildings to a central office rather than to connect computers and printers within a campus. Each switching cabinet can handle up to 1,000 customers, although 400-500 is more typical. This neighborhood equipment performs layer 2/layer 3 switching and routing, offloading full layer 3 routing to the carrier's central office. The IEEE 802.3ah standard enables service providers to deliver up to 100 Mbit/s full-duplex over one single-mode optical fiber to the premises depending on the provider. Speeds of 1Gbit/s are becoming commercially available.

#### [edit] Passive optical network

Main article: Passive optical network

A passive optical network (PON) is a point-to-multipoint, fiber to the premises network architecture in which unpowered optical splitters are used to enable a single optical fiber to serve multiple premises, typically 32-128. A PON configuration reduces the amount of fiber and central office equipment required compared with point to point architectures.

Downstream signal coming from the central office is broadcast to each customer premises sharing a fiber. Encryption is used to prevent eavesdropping.

Upstream signals are combined using a multiple access protocol, usually time division multiple access (TDMA). The OLTs "range" the ONUs in order to provide time slot assignments for upstream communication.

### [edit] Electrical portion

Once on private property, the signal typically travels the final distance to the end user's equipment using an electrical format.

A device called an Optical Network Terminal (ONT), also called an Optical Network Unit (ONU), converts the optical signal into an electrical signal. (ONT is an ITU-T term, whereas ONU is an IEEE term, but the two terms mean exactly the same thing.) Optical network terminals require electrical power for their operation, so some providers connect them to back-up batteries in case of power outages. Optical network units use thin film filter technology to convert between optical and electrical signals.

For fiber to the home and for some forms of fiber to the building, it is common for the building's existing phone systems, local area networks, and cable TV systems to connect directly to the ONT.

If all three systems cannot directly reach the ONT, it is possible to combine signals and transport them over a common medium. Once closer to the end-user, equipment such as a router, modem, and/or network interface module can separate the signals and convert them into the appropriate protocol. For example, one solution for apartment buildings uses VDSL to combine data (and / or video) with voice. With this approach, the combined signal travels through the building over the existing telephone wiring until it reaches the end-user's living space. Once there, a VDSL modem copies the data and video signals and converts them into Ethernet protocol. These are then sent over the end user's category 5 cable. A network interface module can then separate out the video signal and convert it into an RF signal that is sent over the end-user's coaxial cable. The voice signal continues to travel over the phone wiring and is sent through DSL filters to remove the video and data signals. An alternative strategy allows data and / or voice to be transmitted over coaxial cable. In yet another strategy, some office buildings dispense with the telephone wiring altogether, instead using voice over Internet Protocol phones that can plug directly into the local area network.

## [edit] See also

- Broadband Internet access

- Fiber-optic communication

- Fiber to the premises by country

- Next generation access

- Hybrid fiber-coaxial

- Fiber in the loop

## [edit]